Pre-Final Test

Test 1

Date: 2021/04/06

Tester: Lide Cui

Author: Shichuang Zhang, Junjian Chen

Hardware version: 1.5 (in Part 2.5 of Hardware Document)

Software version: 2.0 (in Part 7.0 of Software Document)

Test Purpose:

Test the performance of the robot in a simulated map.

Test Procedure:

Disable server, import map data from xml file and run the robot.

- Robot will be placed at a red or green island by location.

- Given time limit of 5 minutes, observe performance of the robot in following parts:

- localization

- across bridge

- traverse waypoints

- avoid obstacle

- go up and go under the overpass

- return home

Test Data:

detailed data can be found in example_data_file.xml

Expected Result:

| Task | Comment |
| --- | --- |
| localization | The robot should identify itself at green corner |
| across bridge | The robot should go from green corner to island |
| traverse waypoints | The robot can traverse each each point |
| avoid obstacles | The robot should avoid obstacles |
| go up and go under overpass | The robot should identify special point to go up or go under the overpass |
| return home | The robot should be able to return home after one iteration. |

Test Results:

| Tasks | Result |
| --- | --- |
| localization | The robot identified itself at (14, 8) |
| across bridge | The robot crossed bridge from (14.5, 7.5) to (14.5, 4.5) |
| traverse waypoints | The robot rearranged the waypoints and traversed (14, 4), (11, 5) in sequence |
| avoid obstacles | The robot dropped into the sea when it tried to avoid obstacle between (11,5) and (8,4) |
| go up and go under overpass | Not able to test |
| return home | Not able to test |

Test Report:

The robot failed the pre-final tests, due to its incapability of avoiding obstacles. It fell into the sea due to turning left first, but it could not identify the sea.

Conclusion:

Tests do not pass due to obstacle avoidance

Action:

- Revise the obstacle avoidance: When calculating a new path which avoids the obstacle, the algorithm should check whether the new points on the path belong to the island. If not, it should try to calculate a different path.

Distribution: Software development

Test 2

Date: 2021/4/11

Tester: Junjian Chen

Author: Junjian Chen

Hardware version: 1.5 (in Part 2.5 of Hardware Document)

Software version: 2.3 (in Part 7.0 of Software Document)

Test Purpose:

Test whether the robot is able to pass the tunnel from its start point.

Test Procedure:

- Place the robot on 45 degree line at the corner

- Start the simulation

- Observe whether the robot is able to pass the tunnel, record in which part the robot passes or fails.

- If the robot completes the whole process successfully, calculate the error distance

- Repeat each trial for 5 times

Test Data/Expected Result:

Testing Map for Trial 1-5,6-10:

Testing Map for Trial 11-15,16-20:

The robot is able to:

- Finish ultrasonic/light localization through the yellow path

- Finish the tunnel passing through the red path

- Finish after-tunnel light localization through the blue path

Test Results:

| Trial# | Fail/Pass | Expected Final Position/(x,y) | Actual Final Position/(x,y) | Error Distance/Feet |
| --- | --- | --- | --- | --- |
| 1 | Pass | (1,4) | (0.98,3.98) | 0.028 |
| 2 | Pass |  | (0,97,3.98) | 0.036 |
| 3 | Pass |  | (0.96,4.03) | 0.05 |
| 4 | Pass |  | (1.01,4.01) | 0.014 |
| 5 | Pass |  | (1.02,4) | 0.02 |
| 6 | Pass | (14,4) | (14.01,4.01) | 0.014 |
| 7 | Pass |  | (14.01,4.01) | 0.014 |
| 8 | Pass |  | (14.04,3.99) | 0.041 |
| 9 | Pass |  | (13.99,3.99) | 0.014 |
| 10 | Pass |  | (13.97,4.02) | 0.036 |
| 11 | Pass | (5,1) | (5.01,1.01) | 0.014 |
| 12 | Pass |  | (4.98,0.99) | 0.022 |
| 13 | Pass |  | (5.05,1.03) | 0.058 |
| 14 | Pass |  | (5.02,1.04) | 0.045 |
| 15 | Pass |  | (4.99,1.01) | 0.014 |
| 16 | Pass | (10,8) | (10,7.96) | 0.04 |
| 17 | Pass |  | (10.01,8.02) | 0.022 |
| 18 | Pass |  | (9.98,7.99) | 0.022 |
| 19 | Pass |  | (9.97,8.03) | 0.03 |
| 20 | Pass |  | (10.02,8.01) | 0.022 |

Test Report:

Pass Rate:100%

Average Error Distance: 0.0271

All trials pass the test and the average error distance is within acceptance.

Conclusion: The test passes.

Action: None

Distribution: Software Development